	Revision History: G1000 Guided Scenarios Instructor Checklist, Civil Air Patrol, Jeffco Squadron

	Date
	Who
	ID
	Revisions

	11 Dec 09
	1st Lt Todd Gamber CAPID 443746
	v1
	· Initial Revision. Included only cockpit configuration guidelines.

	12 Mar 10

Next Revision Expected 12 Jul 10
	1st Lt Todd Gamber CAPID 443746
	v2
	· Changed to two-column layout and landscape orientation for kneeboard-like formatting.

· Moved time limits and scenario-specific information from the student checklist to the instructor checklist. This generalizes the student checklist so it can be used in actual missions.


Instructor checklist for G1000 Observer scenario-based training

Beginner
· Place audio panel on auto-squelch

· Place Comm volumes at a reasonable volume

· Toggle the seat selector switch to transmit from seat 2

· Toggle the transmit band switch to use FM

· Switch FM radio to channel 1

· Place Becker DF on 121.775 MHz

Intermediate
· Place audio panel on manual squelch at a reasonable squelch level

· Place Comm volumes at a slightly high volume
· Toggle the seat selector switch to transmit from seat 3

· Toggle the transmit band switch to use FM

· Switch FM radio to channel 31

· Place Becker DF on 121.500 MHz

Advanced
· Place audio panel on manual squelch at a very high squelch level (so the intercom doesn’t work at a normal voice level)

· Place Comm volumes at about 2%

· Toggle the seat selector switch to transmit from seat 2

· Toggle the transmit band switch to use VHF

· Switch FM radio off (regardless of mission master switch position)
· Place Becker DF on 121.500 MHz
Scenario: ELT heard at BRK 023 radial at 31 miles. Depart KBJC, go via HOHUM intersection. Perform a grid search of DEN 83B and listen for ELT. Return to KBJC via CASSE NDB.

Evaluate the student on the following criteria:

	During Preflight
	10 minutes or less

Ensure all four corners are entered in the flight plan:
NW corner: W104° 22.5’, N39° 30’
NE corner: W104° 15’, N39° 30’
SW corner: W104° 22.5’, N39° 22.5’
SE corner: W104° 15’, N39° 22.5’
Extra credit: Waypoint at BRK R023 at 31NM

Ensure Becker DF is tuned to proper frequency

	Before Taxi
	2 minutes or less

Ensure audio panel is configured correctly
Ensure first FPL leg is active (or at least entered)

	Before Takeoff
	Ensure student records time off using PFD

	After Takeoff
	Ensure student knows how to configure the FM radio and audio panel to call mission base

	En route
	10 minutes or less

Set up a grid search with North-South tracks. Use appropriate leg length and number of legs



	In Grid
	If available, have someone activate a practice beacon, and note the desired heading to intercept the practice beacon.
Simulate a point of interest inside the grid. Ensure student marks, save, and write down any point in Grid 89B.



